CONCRETE BEAM BRIDGE DECK GRADES

The following information provides an example of the use of an Excel spreadsheet program for the calculation of bridge deck grades on concrete beam bridges.  This example is in Metric units, but the same approach would be followed for use of the English bridge deck grade spreadsheets.

Excel spreadsheet programs for calculation of bridge deck grades for concrete beam bridges is available on the Iowa DOT web site in both Metric and English unit formats.

The following is an example view of the Excel spreadsheet for calculation of bridge deck grades for concrete beam bridges.

| Project No.: |  |  |  |  |  | County: |  |  |  | Page: |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contract ID: |  |  |  |  |  | Design: |  |  |  |  |  |
| Span |  | Beam | Beam | Beam | Beam | Beam | Beam | Beam | Beam | Beam | Beam |
| Lane | Shot |  |  |  |  |  |  |  |  |  |  |
| Location 2 | 15 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 14 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 13 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 12 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 11 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 10 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 9 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 8 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 7 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 6 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 5 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 4 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 3 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 2 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Location 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

| Span |  |  | Beam Length ( Total ): |  |  |  | Beam Length ( CL Brg. - CL Brg. ): |  |  |  | Shot Spacing ( mm ): |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Lane | Deck Thickness: |  |  |  | b: |  | a: |  |  |  |  |  |  |  |
| Beam |  |  |  | - Deck | Distance |  |  |  |  |  | Haunch + | Deck | Adjusted |  |
|  |  | Deck |  | Thickness | From | Deflection | Plan Beam |  | Rod | Act. Beam | Embed. - | Adjust. | Haunch + | Deflection |
|  | Shot | Elevation |  | Elevation | CL Brg. | ( m ) | Elev. | H.I. | Reading | Elev. | ( mm ) | ( mm ) | Embed. - | ( mm ) |
| Location 2 | 15 |  |  | 0.000 | 0.00 |  |  |  | 0.000 | 0.000 | NA | 0 | NA |  |
|  | 14 |  |  | 0.000 | 0.00 |  |  |  | 0.000 | 0.000 | NA | 0 | NA |  |
|  | 13 |  |  | 0.000 | 0.00 |  |  |  | 0.000 | 0.000 | NA | 0 | NA |  |
|  | 12 |  |  | 0.000 | 0.00 |  |  |  | 0.000 | 0.000 | NA | 0 | NA |  |
|  | 11 |  |  | 0.000 | 0.00 |  |  |  | 0.000 | 0.000 | NA | 0 | NA |  |
|  | 10 |  |  | 0.000 | 0.00 |  |  |  | 0.000 | 0.000 | NA | 0 | NA |  |
|  | 9 |  |  | 0.000 | 0.00 |  |  |  | 0.000 | 0.000 | NA | 0 | NA |  |
|  | 8 |  |  | 0.000 | 0.00 |  |  |  | 0.000 | 0.000 | NA | 0 | NA |  |
|  | 7 |  |  | 0.000 | 0.00 |  |  |  | 0.000 | 0.000 | NA | 0 | NA |  |
|  | 6 |  |  | 0.000 | 0.00 |  |  |  | 0.000 | 0.000 | NA | 0 | NA |  |
|  | 5 |  |  | 0.000 | 0.00 |  |  |  | 0.000 | 0.000 | NA | 0 | NA |  |
|  | 4 |  |  | 0.000 | 0.00 |  |  |  | 0.000 | 0.000 | NA | 0 | NA |  |
|  | 3 |  |  | 0.000 | 0.00 |  |  |  | 0.000 | 0.000 | NA | 0 | NA |  |
|  | 2 |  |  | 0.000 | 0.00 |  |  |  | 0.000 | 0.000 | NA | 0 | NA |  |
|  | 1 |  |  | 0.000 | 0.00 |  |  |  | 0.000 | 0.000 | NA | 0 | NA |  |
| Location 1 | 0 |  |  | 0.000 | 0.00 |  |  |  | 0.000 | 0.000 | NA | 0 | NA |  |

The following is an example of bridge deck grade calculations for a concrete beam bridge in Metric units.  Copies of the example spreadsheets are shown to illustrate how the information is input and the resulting information from the spreadsheet calculations.

The first spreadsheet, as shown below, is the summary table of haunch and embedment at all survey shot locations on the beams.  This table is automatically calculated after information has been entered on the individual beam grade calculation spreadsheets.  The table shown below is for all beams in Span 2.  This summary will be the information provided to the contractor for setting of formwork and finishing machine.

| Project No.: |  | IX-58-1(46)--3P-07 |  |  |  | County: |  | Black Hawk |  | Page: |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contract ID: |  | 07-0581-046 |  |  |  | Design: |  | 1290 |  |  |  |
| Span 2 |  | Beam | Beam | Beam | Beam | Beam | Beam | Beam | Beam | Beam | Beam |
| S.B.L. | Shot | A | B | C | D | E | F |  |  |  |  |
| Pier 2 | 15 | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
|  | 14 | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
|  | 13 | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
|  | 12 | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
|  | 11 | 64 | 59 | 92 | 80 | 67 | 83 | NA | NA | NA | NA |
|  | 10 | 38 | 28 | 85 | 75 | 62 | 79 | NA | NA | NA | NA |
|  | 9 | 70 | 20 | 76 | 68 | 51 | 67 | NA | NA | NA | NA |
|  | 8 | 57 | 13 | 62 | 55 | 43 | 63 | NA | NA | NA | NA |
|  | 7 | 42 | 14 | 58 | 46 | 33 | 47 | NA | NA | NA | NA |
|  | 6 | 39 | 15 | 48 | 44 | 33 | 34 | NA | NA | NA | NA |
|  | 5 | 40 | 25 | 57 | 52 | 37 | 38 | NA | NA | NA | NA |
|  | 4 | 39 | 35 | 58 | 43 | 42 | 44 | NA | NA | NA | NA |
|  | 3 | 42 | 50 | 64 | 46 | 46 | 48 | NA | NA | NA | NA |
|  | 2 | 55 | 63 | 75 | 58 | 54 | 57 | NA | NA | NA | NA |
|  | 1 | 74 | 83 | 83 | 74 | 67 | 62 | NA | NA | NA | NA |
| Pier 1 | 0 | 93 | 77 | 92 | 71 | 87 | 68 | NA | NA | NA | NA |

The additional spreadsheets are the calculation sheets for each beam in each span of the bridge.  As can be viewed in the following spreadsheets these show the deck grade entries and resulting calculations for beams A – F of Span 2.

Page 1

| Span 2 |  | Beam Length ( Total ): |  |  | 32258 | Beam Length ( CL Brg. - CL Brg. ): |  |  | 31953 | Shot Spacing ( mm ): |  | 3048 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S.B.L. | Deck Thickness: |  | 203 | b: | 95 | a: | 41 |  |  |  |  |  |  |
| Beam |  |  | - Deck | Distance |  |  |  |  |  | Haunch + | Deck | Adjusted |  |
| A |  | Deck | Thickness | From | Deflection | Plan Beam |  | Rod | Act. Beam | Embed. - | Adjust. | Haunch + | Deflection |
|  | Shot | Elevation | Elevation | CL Brg. | ( m ) | Elev. | H.I. | Reading | Elev. | ( mm ) | ( mm ) | Embed. - | ( mm ) |
| Pier 2 | 15 |  | 0.000 | #N/A | #N/A | #N/A | 274.958 | 0.000 | 0.000 | NA | 0 | NA | #N/A |
|  | 14 |  | 0.000 | #N/A | #N/A | #N/A |  | 0.000 | 0.000 | NA | 0 | NA | #N/A |
|  | 13 |  | 0.000 | #N/A | #N/A | #N/A |  | 0.000 | 0.000 | NA | 0 | NA | #N/A |
|  | 12 |  | 0.000 | #N/A | #N/A | #N/A |  | 0.000 | 0.000 | NA | 0 | NA | #N/A |
|  | 11 | 273.491 | 273.288 | 31953.00 |  | 273.288 |  | 1.734 | 273.224 | 64 | 0 | 64 | 0.000 |
|  | 10 | 273.498 | 273.295 | 30480.00 | 0.009 | 273.304 |  | 1.692 | 273.266 | 38 | 0 | 38 | 9.000 |
|  | 9 | 273.516 | 273.313 | 27432.00 | 0.026 | 273.339 |  | 1.689 | 273.269 | 70 | 0 | 70 | 26.000 |
|  | 8 | 273.531 | 273.328 | 24384.00 | 0.039 | 273.367 |  | 1.648 | 273.310 | 57 | 0 | 57 | 39.000 |
|  | 7 | 273.549 | 273.346 | 21336.00 | 0.048 | 273.394 |  | 1.606 | 273.352 | 42 | 0 | 42 | 48.000 |
|  | 6 | 273.568 | 273.365 | 18288.00 | 0.053 | 273.418 |  | 1.579 | 273.379 | 39 | 0 | 39 | 53.000 |
|  | 5 | 273.586 | 273.383 | 15240.00 | 0.054 | 273.437 |  | 1.561 | 273.397 | 40 | 0 | 40 | 54.000 |
|  | 4 | 273.607 | 273.404 | 12192.00 | 0.051 | 273.455 |  | 1.542 | 273.416 | 39 | 0 | 39 | 51.000 |
|  | 3 | 273.629 | 273.426 | 9144.00 | 0.044 | 273.470 |  | 1.530 | 273.428 | 42 | 0 | 42 | 44.000 |
|  | 2 | 273.647 | 273.444 | 6096.00 | 0.033 | 273.477 |  | 1.536 | 273.422 | 55 | 0 | 55 | 33.000 |
|  | 1 | 273.671 | 273.468 | 3048.00 | 0.019 | 273.487 |  | 1.545 | 273.413 | 74 | 0 | 74 | 19.000 |
| Pier 1 | 0 | 273.693 | 273.490 | 0.00 |  | 273.490 |  | 1.561 | 273.397 | 93 | 0 | 93 | 0.000 |

| Span 2 |  | Beam Length ( Total ): |  |  | 32156 | Beam Length ( CL Brg. - CL Brg. ): |  |  | 31852 | Shot Spacing ( mm ): | 3048 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S.B.L. | Deck Thickness: |  | 203 | b: | 95 | a: | 43 |  |  |  |  |  |  |  |
| Beam |  |  | - Deck | Distance |  |  |  |  |  | Haunch + | Deck | Adjusted |  |  |
| B |  | Deck | Thickness | From | Deflection | Plan Beam |  | Rod | Act. Beam | Embed. - | Adjust. | Haunch + | Deflection |  |
|  | Shot | Elevation | Elevation | CL Brg. | ( m ) | Elev. | H.I. | Reading | Elev. | ( mm ) | ( mm ) | Embed. - | ( mm ) |  |
| Pier 2 | 15 |  | 0.000 | #N/A | #N/A | #N/A | 274.958 | 0.000 | 0.000 | NA | 0 | NA | #N/A |  |
|  | 14 |  | 0.000 | #N/A | #N/A | #N/A |  | 0.000 | 0.000 | NA | 0 | NA | #N/A |  |
|  | 13 |  | 0.000 | #N/A | #N/A | #N/A |  | 0.000 | 0.000 | NA | 0 | NA | #N/A |  |
|  | 12 |  | 0.000 | #N/A | #N/A | #N/A |  | 0.000 | 0.000 | NA | 0 | NA | #N/A |  |
|  | 11 | 273.379 | 273.176 | 31852.00 |  | 273.176 |  | 1.841 | 273.117 | 59 | 0 | 59 | 0.000 |  |
|  | 10 | 273.385 | 273.182 | 30480.00 | 0.009 | 273.191 |  | 1.795 | 273.163 | 28 | 0 | 28 | 9.000 |  |
|  | 9 | 273.403 | 273.200 | 27432.00 | 0.025 | 273.225 |  | 1.753 | 273.205 | 20 | 0 | 20 | 25.000 |  |
|  | 8 | 273.418 | 273.215 | 24384.00 | 0.037 | 273.252 |  | 1.719 | 273.239 | 13 | 0 | 13 | 37.000 |  |
|  | 7 | 273.437 | 273.234 | 21336.00 | 0.046 | 273.280 |  | 1.692 | 273.266 | 14 | 0 | 14 | 46.000 |  |
|  | 6 | 273.455 | 273.252 | 18288.00 | 0.051 | 273.303 |  | 1.670 | 273.288 | 15 | 0 | 15 | 51.000 |  |
|  | 5 | 273.473 | 273.270 | 15240.00 | 0.052 | 273.322 |  | 1.661 | 273.297 | 25 | 0 | 25 | 52.000 |  |
|  | 4 | 273.495 | 273.292 | 12192.00 | 0.049 | 273.341 |  | 1.652 | 273.306 | 35 | 0 | 35 | 49.000 |  |
|  | 3 | 273.516 | 273.313 | 9144.00 | 0.043 | 273.356 |  | 1.652 | 273.306 | 50 | 0 | 50 | 43.000 |  |
|  | 2 | 273.537 | 273.334 | 6096.00 | 0.032 | 273.366 |  | 1.655 | 273.303 | 63 | 0 | 63 | 32.000 |  |
|  | 1 | 273.559 | 273.356 | 3048.00 | 0.018 | 273.374 |  | 1.667 | 273.291 | 83 | 0 | 83 | 18.000 |  |
| Pier 1 | 0 | 273.583 | 273.380 | 0.00 |  | 273.380 |  | 1.655 | 273.303 | 77 | 0 | 77 | 0.000 |  |

Page 2

| Span 2 |  | Beam Length ( Total ): |  |  |  | 32055 | Beam Length ( CL Brg. - CL Brg. ): |  |  |  | 31750 | Shot Spacing ( mm ): |  |  | 3048 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S.B.L. | Deck Thickness: |  | 203 | b: | 95 |  | a: | 44 |  |  |  |  |  |  |  |  |  |  |
| Beam |  |  | - Deck | Distance |  |  |  |  |  |  |  | Haunch + | Deck |  |  | Adjusted |  |  |
| C |  | Deck | Thickness | From | Deflection |  | Plan Beam |  | Rod |  | Act. Beam | Embed. - | Adjust. |  |  | Haunch + |  | Deflection |
|  | Shot | Elevation | Elevation | CL Brg. | ( m ) |  | Elev. | H.I. | Reading |  | Elev. | ( mm ) | ( mm ) |  |  | Embed. - |  | ( mm ) |
| Pier 2 | 15 |  | 0.000 | #N/A | #N/A |  | #N/A |  | 0.000 |  | 0.000 | NA | 0 |  |  | NA |  | #N/A |
|  | 14 |  | 0.000 | #N/A | #N/A |  | #N/A |  | 0.000 |  | 0.000 | NA | 0 |  |  | NA |  | #N/A |
|  | 13 |  | 0.000 | #N/A | #N/A |  | #N/A |  | 0.000 |  | 0.000 | NA | 0 |  |  | NA |  | #N/A |
|  | 12 |  | 0.000 | #N/A | #N/A |  | #N/A |  | 0.000 |  | 0.000 | NA | 0 |  |  | NA |  | #N/A |
|  | 11 | 273.266 | 273.063 | 31750.00 |  |  | 273.063 |  | 0.000 |  | 272.971 | 92 | 0 |  |  | 92 |  | 0.000 |
|  | 10 | 273.272 | 273.069 | 30480.00 | 0.008 |  | 273.077 |  | 0.000 |  | 272.992 | 85 | 0 |  |  | 85 |  | 8.000 |
|  | 9 | 273.290 | 273.087 | 27432.00 | 0.024 |  | 273.111 |  | 0.000 |  | 273.035 | 76 | 0 |  |  | 76 |  | 24.000 |
|  | 8 | 273.306 | 273.103 | 24384.00 | 0.036 |  | 273.139 |  | 0.000 |  | 273.077 | 62 | 0 |  |  | 62 |  | 36.000 |
|  | 7 | 273.324 | 273.121 | 21336.00 | 0.045 |  | 273.166 |  | 0.000 |  | 273.108 | 58 | 0 |  |  | 58 |  | 45.000 |
|  | 6 | 273.342 | 273.139 | 18288.00 | 0.050 |  | 273.189 |  | 0.000 |  | 273.141 | 48 | 0 |  |  | 48 |  | 50.000 |
|  | 5 | 273.363 | 273.160 | 15240.00 | 0.051 |  | 273.211 |  | 0.000 |  | 273.154 | 57 | 0 |  |  | 57 |  | 51.000 |
|  | 4 | 273.382 | 273.179 | 12192.00 | 0.048 |  | 273.227 |  | 0.000 |  | 273.169 | 58 | 0 |  |  | 58 |  | 48.000 |
|  | 3 | 273.403 | 273.200 | 9144.00 | 0.042 |  | 273.242 |  | 0.000 |  | 273.178 | 64 | 0 |  |  | 64 |  | 42.000 |
|  | 2 | 273.424 | 273.221 | 6096.00 | 0.032 |  | 273.253 |  | 0.000 |  | 273.178 | 75 | 0 |  |  | 75 |  | 32.000 |
|  | 1 | 273.446 | 273.243 | 3048.00 | 0.018 |  | 273.261 |  | 0.000 |  | 273.178 | 83 | 0 |  |  | 83 |  | 18.000 |
| Pier 1 | 0 | 273.467 | 273.264 | 0.00 |  |  | 273.264 |  | 0.000 |  | 273.172 | 92 | 0 |  |  | 92 |  | 0.000 |
| Span 2 |  | Beam Length ( Total ): |  |  |  | 31953 | Beam Length ( CL Brg. - CL Brg. ): |  |  |  | 31648 | Shot Spacing ( mm ): | 3048 |  |  |  |  |  |
| S.B.L. | Deck Thickness: |  | 203 | b: | 95 |  | a: | 44 |  |  |  |  |  |  |  |  |  |  |
| Beam |  |  | - Deck | Distance |  |  |  |  |  |  |  | Haunch + | Deck |  |  | Adjusted |  |  |
| D |  | Deck | Thickness | From | Deflection |  | Plan Beam |  | Rod |  | Act. Beam | Embed. - | Adjust. |  |  | Haunch + |  | Deflection |
|  | Shot | Elevation | Elevation | CL Brg. | ( m ) |  | Elev. | H.I. | Reading |  | Elev. | ( mm ) | ( mm ) |  |  | Embed. - |  | ( mm ) |
| Pier 2 | 15 |  | 0.000 | #N/A | #N/A |  | #N/A | 274.958 | 0.000 |  | 0.000 | NA | 0 |  |  | NA |  | #N/A |
|  | 14 |  | 0.000 | #N/A | #N/A |  | #N/A |  | 0.000 |  | 0.000 | NA | 0 |  |  | NA |  | #N/A |
|  | 13 |  | 0.000 | #N/A | #N/A |  | #N/A |  | 0.000 |  | 0.000 | NA | 0 |  |  | NA |  | #N/A |
|  | 12 |  | 0.000 | #N/A | #N/A |  | #N/A |  | 0.000 |  | 0.000 | NA | 0 |  |  | NA |  | #N/A |
|  | 11 | 273.153 | 272.950 | 31648.00 |  |  | 272.950 |  | 2.088 |  | 272.870 | 80 | 0 |  |  | 80 |  | 0.000 |
|  | 10 | 273.159 | 272.956 | 30480.00 | 0.007 |  | 272.963 |  | 2.070 |  | 272.888 | 75 | 0 |  |  | 75 |  | 7.000 |
|  | 9 | 273.178 | 272.975 | 27432.00 | 0.024 |  | 272.999 |  | 2.027 |  | 272.931 | 68 | 0 |  |  | 68 |  | 24.000 |
|  | 8 | 273.193 | 272.990 | 24384.00 | 0.036 |  | 273.026 |  | 1.987 |  | 272.971 | 55 | 0 |  |  | 55 |  | 36.000 |
|  | 7 | 273.211 | 273.008 | 21336.00 | 0.045 |  | 273.053 |  | 1.951 |  | 273.007 | 46 | 0 |  |  | 46 |  | 45.000 |
|  | 6 | 273.229 | 273.026 | 18288.00 | 0.050 |  | 273.076 |  | 1.926 |  | 273.032 | 44 | 0 |  |  | 44 |  | 50.000 |
|  | 5 | 273.251 | 273.048 | 15240.00 | 0.051 |  | 273.099 |  | 1.911 |  | 273.047 | 52 | 0 |  |  | 52 |  | 51.000 |
|  | 4 | 273.269 | 273.066 | 12192.00 | 0.048 |  | 273.114 |  | 1.887 |  | 273.071 | 43 | 0 |  |  | 43 |  | 48.000 |
|  | 3 | 273.290 | 273.087 | 9144.00 | 0.042 |  | 273.129 |  | 1.875 |  | 273.083 | 46 | 0 |  |  | 46 |  | 42.000 |
|  | 2 | 273.312 | 273.109 | 6096.00 | 0.032 |  | 273.141 |  | 1.875 |  | 273.083 | 58 | 0 |  |  | 58 |  | 32.000 |
|  | 1 | 273.333 | 273.130 | 3048.00 | 0.018 |  | 273.148 |  | 1.884 |  | 273.074 | 74 | 0 |  |  | 74 |  | 18.000 |
| Pier 1 | 0 | 273.354 | 273.151 | 0.00 |  |  | 273.151 |  | 1.878 |  | 273.080 | 71 | 0 |  |  | 71 |  | 0.000 |

Page 3

| Span 2 |  | Beam Length ( Total ): |  |  |  | 31852 | Beam Length ( CL Brg. - CL Brg. ): |  |  |  | 31547 | Shot Spacing ( mm ): |  | 3048 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S.B.L. | Deck Thickness: |  | 203 | b: | 95 |  | a: | 44 |  |  |  |  |  |  |  |  |  |
| Beam |  |  | - Deck | Distance |  |  |  |  |  |  |  | Haunch + | Deck |  | Adjusted |  |  |
| E |  | Deck | Thickness | From | Deflection |  | Plan Beam |  | Rod |  | Act. Beam | Embed. - | Adjust. |  | Haunch + |  | Deflection |
|  | Shot | Elevation | Elevation | CL Brg. | ( m ) |  | Elev. | H.I. | Reading |  | Elev. | ( mm ) | ( mm ) |  | Embed. - |  | ( mm ) |
| Pier 2 | 15 |  | 0.000 | #N/A | #N/A |  | #N/A | 274.958 | 0.000 |  | 0.000 | NA | 0 |  | NA |  | #N/A |
|  | 14 |  | 0.000 | #N/A | #N/A |  | #N/A |  | 0.000 |  | 0.000 | NA | 0 |  | NA |  | #N/A |
|  | 13 |  | 0.000 | #N/A | #N/A |  | #N/A |  | 0.000 |  | 0.000 | NA | 0 |  | NA |  | #N/A |
|  | 12 |  | 0.000 | #N/A | #N/A |  | #N/A |  | 0.000 |  | 0.000 | NA | 0 |  | NA |  | #N/A |
|  | 11 | 273.040 | 272.837 | 31547.00 |  |  | 272.837 |  | 2.188 |  | 272.770 | 67 | 0 |  | 67 |  | 0.000 |
|  | 10 | 273.046 | 272.843 | 30480.00 | 0.007 |  | 272.850 |  | 2.170 |  | 272.788 | 62 | 0 |  | 62 |  | 7.000 |
|  | 9 | 273.065 | 272.862 | 27432.00 | 0.023 |  | 272.885 |  | 2.124 |  | 272.834 | 51 | 0 |  | 51 |  | 23.000 |
|  | 8 | 273.080 | 272.877 | 24384.00 | 0.036 |  | 272.913 |  | 2.088 |  | 272.870 | 43 | 0 |  | 43 |  | 36.000 |
|  | 7 | 273.098 | 272.895 | 21336.00 | 0.045 |  | 272.940 |  | 2.051 |  | 272.907 | 33 | 0 |  | 33 |  | 45.000 |
|  | 6 | 273.117 | 272.914 | 18288.00 | 0.050 |  | 272.964 |  | 2.027 |  | 272.931 | 33 | 0 |  | 33 |  | 50.000 |
|  | 5 | 273.138 | 272.935 | 15240.00 | 0.051 |  | 272.986 |  | 2.009 |  | 272.949 | 37 | 0 |  | 37 |  | 51.000 |
|  | 4 | 273.156 | 272.953 | 12192.00 | 0.048 |  | 273.001 |  | 1.999 |  | 272.959 | 42 | 0 |  | 42 |  | 48.000 |
|  | 3 | 273.178 | 272.975 | 9144.00 | 0.042 |  | 273.017 |  | 1.987 |  | 272.971 | 46 | 0 |  | 46 |  | 42.000 |
|  | 2 | 273.199 | 272.996 | 6096.00 | 0.032 |  | 273.028 |  | 1.984 |  | 272.974 | 54 | 0 |  | 54 |  | 32.000 |
|  | 1 | 273.220 | 273.017 | 3048.00 | 0.018 |  | 273.035 |  | 1.990 |  | 272.968 | 67 | 0 |  | 67 |  | 18.000 |
| Pier 1 | 0 | 273.242 | 273.039 | 0.00 |  |  | 273.039 |  | 2.006 |  | 272.952 | 87 | 0 |  | 87 |  | 0.000 |
| Span 2 |  | Beam Length ( Total ): |  |  |  | 31750 | Beam Length ( CL Brg. - CL Brg. ): |  |  |  | 31445 | Shot Spacing ( mm ): | 3048 |  |  |  |  |
| S.B.L. | Deck Thickness: |  | 203 | b: | 95 |  | a: | 49 |  |  |  |  |  |  |  |  |  |
| Beam |  |  | - Deck | Distance |  |  |  |  |  |  |  | Haunch + | Deck |  | Adjusted |  |  |
| F |  | Deck | Thickness | From | Deflection |  | Plan Beam |  | Rod |  | Act. Beam | Embed. - | Adjust. |  | Haunch + |  | Deflection |
|  | Shot | Elevation | Elevation | CL Brg. | ( m ) |  | Elev. | H.I. | Reading |  | Elev. | ( mm ) | ( mm ) |  | Embed. - |  | ( mm ) |
| Pier 2 | 15 |  | 0.000 | #N/A | #N/A |  | #N/A | 274.958 | 0.000 |  | 0.000 | NA | 0 |  | NA |  | #N/A |
|  | 14 |  | 0.000 | #N/A | #N/A |  | #N/A |  | 0.000 |  | 0.000 | NA | 0 |  | NA |  | #N/A |
|  | 13 |  | 0.000 | #N/A | #N/A |  | #N/A |  | 0.000 |  | 0.000 | NA | 0 |  | NA |  | #N/A |
|  | 12 |  | 0.000 | #N/A | #N/A |  | #N/A |  | 0.000 |  | 0.000 | NA | 0 |  | NA |  | #N/A |
|  | 11 | 272.931 | 272.728 | 31445.00 |  |  | 272.728 |  | 2.313 |  | 272.645 | 83 | 0 |  | 83 |  | 0.000 |
|  | 10 | 272.934 | 272.731 | 30480.00 | 0.005 |  | 272.736 |  | 2.301 |  | 272.657 | 79 | 0 |  | 79 |  | 5.000 |
|  | 9 | 272.952 | 272.749 | 27432.00 | 0.020 |  | 272.769 |  | 2.256 |  | 272.702 | 67 | 0 |  | 67 |  | 20.000 |
|  | 8 | 272.967 | 272.764 | 24384.00 | 0.032 |  | 272.796 |  | 2.225 |  | 272.733 | 63 | 0 |  | 63 |  | 32.000 |
|  | 7 | 272.986 | 272.783 | 21336.00 | 0.040 |  | 272.823 |  | 2.182 |  | 272.776 | 47 | 0 |  | 47 |  | 40.000 |
|  | 6 | 273.004 | 272.801 | 18288.00 | 0.045 |  | 272.846 |  | 2.146 |  | 272.812 | 34 | 0 |  | 34 |  | 45.000 |
|  | 5 | 273.025 | 272.822 | 15240.00 | 0.046 |  | 272.868 |  | 2.128 |  | 272.830 | 38 | 0 |  | 38 |  | 46.000 |
|  | 4 | 273.043 | 272.840 | 12192.00 | 0.044 |  | 272.884 |  | 2.118 |  | 272.840 | 44 | 0 |  | 44 |  | 44.000 |
|  | 3 | 273.065 | 272.862 | 9144.00 | 0.038 |  | 272.900 |  | 2.106 |  | 272.852 | 48 | 0 |  | 48 |  | 38.000 |
|  | 2 | 273.086 | 272.883 | 6096.00 | 0.029 |  | 272.912 |  | 2.103 |  | 272.855 | 57 | 0 |  | 57 |  | 29.000 |
|  | 1 | 273.107 | 272.904 | 3048.00 | 0.016 |  | 272.920 |  | 2.100 |  | 272.858 | 62 | 0 |  | 62 |  | 16.000 |
| Pier 1 | 0 | 273.129 | 272.926 | 0.00 |  |  | 272.926 |  | 2.100 |  | 272.858 | 68 | 0 |  | 68 |  | 0.000 |

The following is an explanation of how these spreadsheets are used.  Refer to the spreadsheets above as you read the instructions on use below.

The spreadsheet is set up so each beam in a given span can be calculated individually.  This is very important for a variable skew bridge that has different beam lengths on a given span.

Page 4

If the beams are all the same length on a given span, the information entered for each beam will be the same.

Also note that the plans typically only provide beam deadload deflection information at the midpoint of the beam.  This spreadsheet application will calculate the beam deadload deflection at all elevation shot locations based on the plan spacing of the top of slab elevations and the plan beam midpoint deadload deflection.

If a bridge has three spans, you should have three files, such as:

Span1sbl = span 1 of a southbound lane bridge

Span2sbl = span 2 of a southbound lane bridge

Span3sbl = span 3 of a southbound lane bridge

Within the spreadsheet, the cells that are unprotected are color coded so information can be entered.

Page 1 of the example is the table printed out and given to the contractor for each span of the bridge.  The following information needs to be entered in the color coded cells.

Project Number – listed on the plans

Contract ID Number – listed on the contract

County – listed on the plans

Design Number – listed on the plans

Span – enter the span the information represents, ex. Span 2 – span 10

Lane – enter the lane to be represented, ex. s.b.l. – n.b.l. – w.b.l.

Location at beginning of table – enter the location of the end of the span,

ex. Pier 2.

Location at bottom of table – enter the location of the start of the span,

Ex. Pier 1

Beam – enter the letter of each beam in the span

Once this information has been entered, each beam can be calculated.  Press the  Pg Dn  key on your keyboard or arrow down the screen to find the tables for each beam.

To calculate the proper grades, information is taken from the plans for each beam on a given span.  Enter this information in the color coded cells provided, refer to pages 2, 3, and 4 of the example.  On page 2 of the example the following information was entered for Beam A of Span 2 between Piers 1 & 2.

Beam Length (Total):		32258 mm

Beam Length (CL Brg. – CL Brg.):		31953 mm

Shot Spacing:		  3048 mm

Deck Thickness:		    203 mm

As Erected b:		      95 mm

Slab In Place a:		      41 mm

The next piece of information needed is the Deck Elevations, which are listed on the plans.  The elevations are entered in meters, refer to page 2 of the example for Beam A.

Pier 2		15

14

13

12

11	273.491

10	273.498

9	273.516

8	273.531

7	273.549

6	273.568

5	273.586

4	273.607

3	273.629

2	273.647

1	273.671

Pier 1		  0	273.693

After the deck elevations are entered, the computer will subtract the deck thickness and add the proper deflection to each elevation.  This elevation is under the Plan Beam Elev. column.

There are two ways to enter the Act. Beam Elev. figures.  The first way is to enter the H.I. of the instrument and each individual rod reading.  The computer will subtract each rod reading from the H.I. figure.  The other method is by using a direct reading rod and entering the elevations directly into the Act. Beam Elev. column.  This column is unprotected even though it is not color coded, refer to page 3 Beam C for an example of the elevations being entered directly.

The computer will calculate the proper haunch or embedment by subtracting the Act. Beam Elev. from the Plan Beam Elev. figure.

The Deck Adjustment column is only used in a case where the deck grades have to be adjusted because the resulting haunch or embedment exceeds what is permitted by the Plan.  This may be caused by beams being over cambered, under cambered, or beam seat elevations being high or low.

A bridge can be set up before the project even starts by entering the beam information and deck elevations for each span.  After the beams are set into place, each span file can be opened and the actual beam elevations entered.